# Project Report

# (CSE3010) COMPUTER VISION

# Winter Semester:2025-2026

# NAME: ARADHAYA SINGH

# REG. NO. 23BAI10892

# TOPIC:    DRIVER DROWSINESS                     DETECTION SYSTEM

# 📌 1. Problem Statement

Road accidents often occur due to driver fatigue and drowsiness. Detecting early signs of drowsiness can help prevent accidents and improve road safety.

# 🎯 2. Objective

To build a real-time system that:

- Detects a driver's face and eyes using a webcam

- Monitors eye closure (blink rate / eye aspect ratio)

- Triggers an alert when drowsiness is detected

# 🧠 3. Key Concept Used

- Face Detection (Haar Cascade / Dlib)

- Eye Detection

- Facial Landmark Detection

- Eye Aspect Ratio (EAR)

- Real-time Video Processing (OpenCV)

# ⚙️ 4. System Architecture

Webcam Input → Face Detection → Eye Detection → EAR Calculation → Drowsiness Check → Alert

# 🔍 5. Methodology (CORE PART FOR MARKS)

## Step 1: Capture Video

- Use OpenCV to access webcam

## Step 2: Detect Face

- Haar Cascade classifier

## Step 3: Detect Eyes / Landmarks

- Use dlib facial landmarks (68 points)

## Step 4: Compute Eye Aspect Ratio (EAR)

👉 This is the most important formula:

EAR=∣∣p2−p6∣∣+∣∣p3−p5∣∣2∣∣p1−p4∣∣EAR = \frac{||p_2 - p_6|| + ||p_3 - p_5||}{2||p_1 - p_4||}EAR=2∣∣p1​−p4​∣∣∣∣p2​−p6​∣∣+∣∣p3​−p5​∣∣​

Where:

- p1 to p6 are eye landmark points

👉 Interpretation:

- EAR ≈ constant → eyes open

- EAR ↓ → eyes closing

- EAR < threshold → drowsy

## Step 5: Drowsiness Detection Logic

- If EAR < 0.25 for continuous frames (e.g., 20 frames)
→ Trigger alert

## Step 6: Alert System

- Sound alarm (beep)

- Display warning on screen

🧠 CODE:

import cv2

import dlib

import numpy as np

from scipy.spatial import distance

def calculate_EAR(eye):

A = distance.euclidean(eye[1], eye[5])

B = distance.euclidean(eye[2], eye[4])

C = distance.euclidean(eye[0], eye[3])

return (A + B) / (2.0 * C)

# Initialize

detector = dlib.get_frontal_face_detector()

predictor = dlib.shape_predictor("shape_predictor_68_face_landmarks.dat")

cap = cv2.VideoCapture(0)

EAR_THRESHOLD = 0.25

FRAME_COUNT = 20

counter = 0

while True:

ret, frame = cap.read()

if not ret:

print("Camera error")

break

gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)

faces = detector(gray)

for face in faces:

landmarks = predictor(gray, face)

left_eye = []

right_eye = []

for i in range(36, 42):

left_eye.append((landmarks.part(i).x, landmarks.part(i).y))

for i in range(42, 48):

right_eye.append((landmarks.part(i).x, landmarks.part(i).y))

leftEAR = calculate_EAR(left_eye)

rightEAR = calculate_EAR(right_eye)

EAR = (leftEAR + rightEAR) / 2.0

if EAR < EAR_THRESHOLD:

counter += 1

if counter >= FRAME_COUNT:

cv2.putText(frame, "DROWSINESS ALERT!", (50, 100),

cv2.FONT_HERSHEY_SIMPLEX, 1, (0, 0, 255), 3)

else:

counter = 0

cv2.imshow("Drowsiness Detection", frame)

if cv2.waitKey(1) & 0xFF == 27:

break

cap.release()

cv2.destroyAllWindows()

# 💻 WHEN IT WORKS IN VS CODE

You will see:

### 🟢 Normal:

- Webcam opens

- Face visible

## 6. Results

- Successfully detects face and eyes

- Identifies drowsiness in real-time

- Displays alert on screen

### 4.3 Tools & Technologies

- Python

- OpenCV

- dlib

- NumPy

- SciPy

## 5. Implementation

The system uses:

- dlib for face detection and landmark prediction

- OpenCV for video processing

- EAR calculation for drowsiness detection

When EAR stays below threshold for multiple frames, an alert message is displayed.

## 7. Challenges Faced

- Low lighting reduces accuracy

- Glasses may affect detection

- Camera positioning is important